APPROVED FAN STOPPAGE PLAN

COMPLIANCE WITH KRS 352.030

THIS PLAN SUPERSEDES ALL PREVIOUS FAN STOPPAGE PLANS FOR THIS MINE.

| LICENSEE |  |
| --- | --- |

| MINE NUMBER |  |
| --- | --- |

| STATE FILE NO |  |
| --- | --- |

| COMPANY ADDRESS |  |
| --- | --- |

(PO BOX OR STREET ADDRESS)

(CITY)                                                                              (STATE)                               (ZIP CODE)

| COMPANY OFFICIAL SIGNATURE |  |
| --- | --- |

| DISTRICT SUPERVISOR |  |
| --- | --- |

| DISTRICT INSPECTOR |  |
| --- | --- |

| RECOMMENDED APPROVAL |  |
| --- | --- |

|  |  | DATE: |  |
| --- | --- | --- | --- |

EB-71

FAN STOPPAGE PLAN

IN ACCORDANCE WITH SECTION 352.030 OF THE KENTUCKY REVISED STATUTES THE FOLLOWING PROCEDURE WILL BE FOLLOWED IN EVENT THE FAN(S) STOPS:

1.	ALL MEN WILL BE WITHDRAWN IMMEDIATELY FROM FACE AREAS TO 	A POINT OUTBY THE WORKING SECTION.

2.	ELECTRICAL POWER WILL BE CUT OFF IMMEDIATELY FROM WORKING SECTIONS.

3.	IF VENTILATION IS RESTORED WITHIN 15 MINUTES, THE WORKING PLACES AND ADJACENT AREAS WHERE METHANE OR NOXIOUS GASES MAY HAVE ACCUMULATED WILL BE RE-EXAMINED BY A CERTIFIED OR QUALIFIED PERSON TO DETERMINE IF THE GASES ARE WITHIN ALLOWABLE LIMITS.  THE POWER WILL BE RESTORED AND WORK RESUMED IF ALL PLACES TO BE EXAMINED HAVE BEEN FOUND SAFE.

4.	IF THE FAN IS NOT OPERATING AFTER A PERIOD OF FIFTEEN (15) MINUTES HAVE ELAPSED, ALL MEN WILL BE WITHDRAWN FROM THE MINE AND THE POWER WILL BE TAKEN OFF THE UNDERGROUND WORKINGS EXCEPT FOR POWER NECESSARY TO REMOVE THE MEN FROM THE MINE.  ALL POWER WILL THEN BE CUT OFF AS SOON AS THE MEN ARE OUTSIDE.

5.	WHEN VENTILATION IS RESTORED AND THE VENTILATION PRESSURE IS NORMAL, THE ENTIRE MINE WILL BE EXAMINED BY CERTIFIED OR QUALIFIED PERSONS.  WHEN THE EXAMINATION IS COMPLETED AND 	WORKINGS ARE FOUND TO BE SAFE FOR RE-ENTRY, THE POWER WILL BE RESTORED AND MEN WILL RETURN TO THEIR PLACES TO WORK.

6.	IF AN AUXILIARY STANDBY ENGINE IS PROVIDED, IT WILL BE USED ONLY AS AN ADDITIONAL SAFEGUARD TO FACILITATE THE WITHDRAWAL OF MEN FROM THE MINE IN EVENT THE PRIMARY FAN POWER SOURCE IS INTERRUPTED.