# Kentucky Explosives and Blasting Branch

Suggested Blaster Training Topics

2016

- Shots without relief

  - No relief to the any side means the only relief is upward

  - Only upward relief means a highly elevated risk of vertical flyrock

  - Modification of blast designs are required

- Wet conditions

  - Increased risk of flyrock during and after long periods of extremely wet weather

  - Effects of water in a blast hole, i.e. increased specific gravity of muddy water

  - Effects of water on stemming material

  - Effects of water on earthen material near a blast hole

- Decoupled explosives

  - Distribute energy differently than do coupled explosives loads.

  - Hydraulic effects of water in bore holes are increased by decoupled explosives

  - Pre-split parameters magnify these problems

- Unique Geology

  - Especially some Limestone formations and conditions

  - The blasting expert is challenged to figure out how to safely and effectively blast unique geology (Blaster=licensed professional)

- Seismographs

- Proper placement, geophone / microphone

- Uploading

- Data analysis

- Misfires / Cutoffs

- Blaster should STOP and THINK before acting

- Blaster should seek advice of his supervisor / tech rep

- Blast now has new parameters, burdens should be measured

- New hazards for flyrock have been created

- The blast area to be, at minimum,  doubled in size

- The blast should be re-fired only if ground conditions surrounding the undetonated explosives have a reasonable expectation of preventing flyrock

- Blasting mats

  - Highly encouraged when blasting near homes and/or protected structures

  - Critical when any of the above conditions are present